NLD SYNAPSE 2026  ·  DECODE X  ·  CASE GRIDSHIELD

GRIDSHIELD

Stage 3: Confidential Board Directive

Constraint Compliance, Risk Transparency & Trade-off Articulation

01 March 2026, 9:00 AM  |  Lumina Energy, Suburban Mumbai

| Total Penalty | Peak Penalty | C3 Bias | C4 Uplift |
| --- | --- | --- | --- |
| Rs. 1,83,336 | Rs. 31,626 | +1.278% ✓ | +1.008% ✓ |
| S2 Baseline | Improvement | C2 Violations | All 26 |
| Rs. 1,89,033 | −Rs. 5,697 | 26 intervals | Force-Majeure |

# 1. Executive Summary

Stage 3 presents six binding constraints issued by the Board following review of Stage 2 results and grid sensitivity concerns. This report provides full compliance assessment for each constraint, supported by empirical evidence and mathematical justification. The central finding is this: three constraints — C1 (financial exposure cap), C3 (bias bound), and C4 (buffering constraint) — are satisfied by our Stage 3 Adaptive Floor strategy. One constraint — C2 (peak reliability: ≤3 intervals of >5% under-forecast at peak) — cannot be satisfied simultaneously with C3 and C4 under the demand conditions of the May 2021 test period. This is not a model failure; it is a mathematical consequence of the Board issuing conflicting requirements in the presence of a force-majeure demand regime.

The Stage 3 Adaptive Floor strategy reduces total penalty from Rs. 1,89,033 (Stage 2 baseline) to Rs. 1,83,336 — a Rs. 5,697 improvement. Peak-hour penalty drops by 15.2% from Rs. 37,323 to Rs. 31,626. The remaining 26 peak-reliability violations are fully attributed to two verifiable structural events: the Break the Chain unlock transition on May 3-4 and the post-Cyclone Tauktae demand rebound on May 24-25. The transparent trade-off between C2 and the C3/C4 bounds is quantified, proven by exhaustive search, and reported in compliance with the Board's C6 executive expectation.

| Constraint | Stage 3 Result | Board Bound | Status |
| --- | --- | --- | --- |
| C1: Total Penalty | Rs. 1,83,336 | < Rs. 1,89,033 | ✓ PASS |
| C2: Peak Violations (>5%) | 26 intervals | ≤ 3 intervals | Force-Majeure |
| C3: Forecast Bias | +1.278% | −2% to +3% | ✓ PASS |
| C4: Avg Uplift vs Mean | +1.008% | ≤ 3% | ✓ PASS |
| C5: Risk Metrics Reported | P95=242.2 kW, Worst 5 | Report only | ✓ REPORTED |
| C6: Trade-off Articulated | C2 vs C3+C4 proven | Transparent disclosure | ✓ DONE |

# 2. Constraint-by-Constraint Analysis

## 2.1 C1 — Total Financial Exposure Cap

The Board defines the threshold as the Stage 2 baseline minus a required improvement margin. Our Stage 2 submission produced a total penalty of Rs. 1,89,033. The Stage 3 Adaptive Floor strategy reduces this to Rs. 1,83,336 — a Rs. 5,697 improvement. The decomposition:

| Component | Stage 2 Hybrid | Stage 3 Adaptive | Δ |
| --- | --- | --- | --- |
| Total Penalty | Rs. 1,89,033 | Rs. 1,83,336 | −5,697 |
| Peak-Hour Penalty | Rs. 37,323 | Rs. 31,626 | −5,697 |
| Off-Peak Penalty | Rs. 1,51,709 | Rs. 1,51,709 | 0 |

The off-peak penalty is identical in both strategies because we do not change the off-peak model (Q67 remains optimal under unchanged off-peak rates). The entire improvement comes from the peak-hour rolling floor, which systematically lifts forecasts during periods when the Q75 model is suppressed by lockdown-era lag features. The Rs. 5,697 improvement is modest relative to the total because the lag contamination problem cannot be fully corrected without actual test-period observations in the lag window.

## 2.2 C2 — Peak-Hour Reliability Constraint (The Binding Trade-off)

This constraint requires that under-forecast deviations exceeding 5% of actual peak load occur in at most 3 intervals across the entire test period. Our Stage 3 strategy achieves 26 such violations — significantly below the 100 violations in the Stage 2 baseline, but still exceeding the 3-interval cap. This section explains precisely why the remaining violations are structurally unavoidable.

Why 26 Violations Remain After Applying the Adaptive Floor

The rolling floor strategy applies a historical percentile floor to peak forecasts starting from Day 3 of the test period, once sufficient observations are available. This eliminates violations caused by random model error. However, 26 violations remain because they arise from two structural discontinuities that no lag-based floor can correct:

- Break the Chain Unlock Transition (May 3–4, 21 violations): The load_t_192 feature for May 3 references April 29, 2021 — a day within the strict Break the Chain lockdown phase, when average load was approximately 1,289 kW. On May 3 (Day 3 of the test period), actual peak load was 1,304 kW because partial commercial activity had resumed on May 1. The 2-day lag is anchored to a demand regime that no longer exists. The rolling floor cannot correct for this because on Day 3, we have only 2 days of test observations — both of which were Saturday and Sunday (low-demand days), providing a floor of ~1,220 kW that is still below the actual weekday peak of ~1,300 kW.

- Post-Cyclone Tauktae Demand Rebound (May 24–25, 5 violations): Cyclone Tauktae suppressed load to 879 kW on May 18. The 7-day lag feature (load_t_672) for May 25 references May 18 — the cyclone nadir. By May 25, load had fully recovered to normal levels (~1,307 kW at peak). Our forecast, anchored partly to May 18 through the 7-day lag, under-predicted the recovery. The rolling floor's 8-day lookback window at this point contains the cyclone-suppressed week, causing the floor itself to be depressed.

The Mathematical Proof That C2 Cannot Coexist with C3 and C4

A constraint conflict exists when the set of models satisfying C2 and the set of models satisfying C3+C4 have no intersection. We demonstrate this by exhaustive parametric search. For a blanket peak uplift factor u applied to the Q75 baseline:

C2 requires: u ≥ 18%  |  C3+C4 require: u ≤ 3%  |  18% > 3% → ∅

In detail: satisfying C2 with ≤3 violations requires an 18% upward shift in peak forecasts (verified by scanning uplift values from 0% to 25%). An 18% uplift applied only to peak hours (which represent 16.7% of all intervals) produces an overall bias of 18% × 16.7% = 3.00% — precisely at the C3 upper bound. It simultaneously produces an average uplift of 3.0% relative to the mean model — precisely at the C4 bound. Any uplift slightly above 3% violates both C3 and C4 simultaneously. The only mathematically feasible uplift range that satisfies both C3 and C4 is 0%–3%, which leaves 26+ violations (far exceeding C2's cap of 3). The constraints are incompatible in the presence of structural demand swings exceeding 10%.

This is the transparent trade-off the Board requested under C6. It is not a forecasting failure — it is an inherent consequence of setting a tight reliability constraint (≤3 miss events in 496 peak intervals = 99.4% coverage) while simultaneously capping the bias and uplift that would be needed to achieve that coverage during a force-majeure demand regime.

## 2.3 C3 — Forecast Bias Bound

The Stage 3 strategy achieves an overall forecast bias of +1.278%. This is within the Board's required range of −2% to +3% and represents a compliant result. The interpretation is important: a bias of +1.278% means our forecasts are, on average, 1.278% above actual load. This is a deliberate and bounded upward bias introduced by the rolling floor, which systematically lifts peak forecasts to buffer against under-forecast risk. The bias does not approach the +3% upper bound and does not suggest over-procurement risk at a material level.

For comparison, the Stage 2 Hybrid strategy had a bias of +0.405% — lower, because it did not apply the upward floor correction. The Stage 3 strategy accepts a higher (but still compliant) bias in exchange for reduced peak under-forecast frequency. This is the correct trade-off under the Rs. 6/kWh under-forecast penalty rate: a small positive bias (over-forecast) costs Rs. 2/kWh, while under-forecast at peak costs Rs. 6/kWh — a 3:1 penalty asymmetry that justifies the upward adjustment.

## 2.4 C4 — Buffering Constraint

The average forecast uplift relative to the unbiased mean model is +1.008% across the entire test window. This is within the Board's 3% cap and represents a modest, well-justified buffering strategy. The uplift arises entirely from the rolling floor applied to peak hours — when the floor is active, it raises the peak forecast above what the Q75 model would otherwise produce. Off-peak forecasts are unchanged (Q67 applied identically to Stage 2). The 1.008% global uplift is below the 3% cap by a factor of 3, providing substantial headroom.

Crucially, the uplift is not constant — it is adaptive. In Week 1 of the test period, when lag features reference full-lockdown April 2021, the floor provides significant upward corrections at peak. As the test-period observations accumulate and the rolling window captures actual unlock-era loads, the floor becomes less binding and the uplift diminishes. This means the strategy is self-regulating: it provides the most correction when it is most needed (early in the test period) and eases off as the lag features become more representative.

# 3. C5 — Risk Transparency

## 3.1 95th Percentile Absolute Deviation

The 95th percentile of absolute deviations between actual and Stage 3 forecast is 242.2 kW. This means that in 95% of intervals, the forecast is within 242.2 kW of actual load. The P95 is marginally lower than the Stage 2 baseline (243.6 kW), confirming that the adaptive floor does not inflate extreme errors. The extreme deviations (beyond P95) are concentrated at the Cyclone Tauktae aftermath intervals — over-forecasts of 500+ kW on May 18, when load collapsed to 879–1,047 kW while our lag-based forecast remained anchored at ~1,520–1,570 kW.

## 3.2 Worst 5 Deviation Intervals

| DateTime | Actual kW | Forecast kW | Deviation kW | Penalty (Rs.) | Root Cause |
| --- | --- | --- | --- | --- | --- |
| 2021-05-18 00:15 | 1,045.9 | 1,567.7 | −521.8 | Rs. 261 | Cyclone Tauktae aftermath: over-forecast |
| 2021-05-18 00:30 | 1,037.7 | 1,553.1 | −515.4 | Rs. 258 | Cyclone Tauktae aftermath: over-forecast |
| 2021-05-18 00:45 | 1,024.9 | 1,540.3 | −515.4 | Rs. 258 | Cyclone Tauktae aftermath: over-forecast |
| 2021-05-18 00:00 | 1,047.2 | 1,562.5 | −515.3 | Rs. 258 | Cyclone Tauktae aftermath: over-forecast |
| 2021-05-18 01:00 | 1,010.5 | 1,520.9 | −510.4 | Rs. 255 | Cyclone Tauktae aftermath: over-forecast |

All five worst intervals occur in the midnight-to-early-morning window on May 18, 2021 (Tuesday). These are all off-peak hours — no C2 violation is associated with them. The deviations are over-forecasts: our model predicted ~1,520–1,570 kW while actual load was ~1,010–1,048 kW, a difference of ~510–522 kW. The root cause is that load_t_192 for these intervals references May 16 (00:00–01:00), when load was at normal levels (~1,500 kW). But by the time May 18 midnight arrived, Cyclone Tauktae had suppressed load dramatically. No 2-day-ahead model — regardless of sophistication — could have predicted a Category 5 cyclone's impact 48 hours in advance.

The financial impact of these five intervals is Rs. 1,290 — a small portion of the total penalty. The large deviation does not translate to proportionally large penalty because these are off-peak over-forecasts, penalised at Rs. 2/kWh.

## 3.3 Financial Impact of Peak-Hour Volatility

Peak hours (6:00 PM – 10:00 PM) represent 496 of 2,977 total test intervals — 16.7% of the time window. Under Stage 3, these intervals generate Rs. 31,626 of the Rs. 1,83,336 total penalty — 17.3% of all penalty. The per-interval penalty cost is Rs. 63.76 at peak versus Rs. 61.15 off-peak, a modest differential despite the much higher under-forecast rate (Rs. 6 vs Rs. 4/kWh). The reason the per-interval cost is not dramatically higher at peak is the upward bias from the rolling floor: by systematically lifting peak forecasts, we shift the error distribution from under-forecast toward over-forecast, which costs only Rs. 2/kWh regardless of peak status. This is the intended effect of the q*=0.750 recalibration combined with the adaptive floor.

## 3.4 Justification of Regime-Adjusted Modeling Strategy

The Stage 3 strategy combines three components, each justified by a specific observed failure mode:

- Q67 for off-peak hours: The off-peak penalty structure (Rs. 4 under, Rs. 2 over) is unchanged from Stage 1. The optimal quantile remains q* = 4/(4+2) = 0.667. Applying any other quantile to off-peak intervals would be sub-optimal in expectation. Off-peak performance is not affected by the Stage 3 regime-adjustment changes.

- Q75 for peak hours: The Stage 2 regulatory change escalated peak under-forecast to Rs. 6/kWh. The newsvendor formula mandates q* = 6/(6+2) = 0.750. This is mathematically derived and cannot be improved upon without violating the optimality condition.

- Rolling 90th-percentile floor for peak hours from Day 3 onward: This addresses the specific lag contamination problem: load_t_192 references April 2021 full-lockdown values (1,289 kW avg) when May 2021 is a partial-unlock regime (1,273 kW avg with different intra-day shape). From Day 3 onward, we observe actual test-period peak loads and compute a floor as the 90th percentile of same-hour loads in the trailing 8-day window. This floor is applied only when it exceeds the Q75 model's output — meaning it only activates when the model is being suppressed by stale lag features. This is temporally valid: all floor values reference observations at least 2 calendar days prior to the forecast interval, consistent with the 2-day-ahead protocol.

The combined strategy satisfies financial prudence (C1), regulatory compliance (C3, C4), and grid stability (bounded positive bias), while providing the maximum achievable reduction in C2 violations within the constraint set. The remaining violations are documented, attributed, and structurally explained — fulfilling the C6 requirement for transparent trade-off articulation.

# 4. Stage 3 Mandatory Metrics

## 4.1 Complete Backtest Results

| Metric | S2 Hybrid | S3 Adaptive | Change | Board Bound |
| --- | --- | --- | --- | --- |
| Total Penalty (Rs.) | 1,89,033 | 1,83,336 | −5,697 | < S2 baseline |
| Peak Penalty (Rs.) | 37,323 | 31,626 | −5,697 | — |
| Off-Peak Penalty (Rs.) | 1,51,709 | 1,51,709 | 0 | — |
| C2 Peak Violations (count) | 100 | 26 | −74 | ≤ 3 |
| C3 Forecast Bias (%) | +0.405% | +1.278% | +0.873% | −2% to +3% |
| C4 Uplift vs Mean (%) | +0.135% | +1.008% | +0.873% | ≤ 3% |
| C5 P95 Abs Deviation (kW) | 243.6 | 242.2 | −1.4 | Report only |
| RMSE (kW) | 116.31 | 119.17 | +2.86 | Report only |
| MAPE (%) | 6.750% | 6.920% | +0.17% | Report only |
| Worst single deviation (kW) | 521.8 | 521.8 | 0 | Report only |

Note on RMSE: The slight increase in RMSE (116.31 → 119.17 kW) from Stage 2 to Stage 3 is expected and acceptable. The rolling floor raises peak forecasts above the Q75 model baseline, which increases RMSE when actual loads remain below the floor. This is the deliberate cost of reducing under-forecast risk. RMSE is not a direct metric in the Board's constraint set — it is reported for completeness. The constraint metrics (C1–C4) all move in the correct direction.

# 5. Adaptive Strategy: Technical Documentation

## 5.1 The Rolling 90th-Percentile Floor

The floor mechanism works as follows. For each peak-hour interval t on day d ≥ 3 of the test period:

- Collect all confirmed actual load observations from peak hours at the same clock hour (e.g., 19:00) in the 2–10 day lookback window. This ensures the floor references only actual data from at least 48 hours prior.

- Compute the 90th percentile of these observations. The 90th percentile is chosen because: (a) it represents a conservative but non-extreme floor, (b) it avoids anchoring to individual outliers, and (c) it provides sufficient upward coverage without violating C4's 3% buffering cap.

- Set the Stage 3 forecast to: max(Q75 model output, 90th-pct floor). The floor is only binding when the Q75 model is suppressed below recent observed levels — i.e., when lag contamination is active.

This mechanism is adaptive in two senses. First, it is temporally adaptive: the floor is not active in the first 2 days (insufficient history) and becomes progressively more representative as more test-period observations enter the window. Second, it is regime-adaptive: as the unlock progresses and loads normalise, the floor naturally incorporates higher actual values, reducing systematic under-forecast bias without any manual intervention.

## 5.2 Why Not a Higher Percentile?

Using a higher percentile (e.g., P95 or P99) would raise more forecasts, potentially reducing C2 violations further. However, it would also increase the uplift vs mean model (C4 risk) and push the overall bias toward the C3 upper bound. The 90th percentile was chosen as the highest floor level that maintains C3 and C4 compliance while maximising C2 violation reduction. This was confirmed by parametric sensitivity analysis.

## 5.3 Temporal Validity and Leakage Check

The strategy uses only observations from at least 2 calendar days before the forecast interval. For an interval on May 5 at 19:00, the lookback window covers May 3 backward — all dates with confirmed actuals. The Q75 model's lag features (load_t_192 = 48-hour lag) are also respected. No test-period actuals are used to construct features for their own forecast interval. The strategy is fully consistent with the 2-day-ahead forecasting protocol.

— End of Stage 3 Report —